# ADC Basic setup + example

# Initialization:

## 1.Enable clock from APB2ENR, ADC1-> pin 9, ADC2->pin 10. AFIO at pin 0.

## 2. Total 16 channels: Find pin and channel number .

If pin is PA (Low 0-7), then pin number is equal channel number.

If pin is PB (0 and 1), then (pin number +8)= channel number.

If pin is PC ( 0 To 5), then (pin number + 10 ) = channel number.

## 3. ADC regular sequence register setup

This this the register for scan sequence of the channels.

ADC->SQR1: sequence of 13 to 16

ADC->SQR2: sequence of 7 to 12

ADC->SQR3: sequence of 0 to 6.

Example we can have a channel sequence like: ch0, ch5, ch1,ch6. These all channel will go to SQR3 from 0 to 3, each slot is 4 bit.

## ADC control register setup

Clear cr1 and cr1 to prevent unwanted configuration

ADC->CR1 =0

ADC->CR2=0;

Continuous conversion: bit 1 (cont)

If we want ADC to convert continuously, not stop after single continuous, need to set CONT = 1.

//before ADON, cont and sqr value must be set.

Power on: bit 0 (ADON)

Then enable ADC by setting bit 0 (ADON). But to start up, we need to powe up bit 0 twice. 1st one do power on, 2nd time start converting.

Example:

ADC->CR2 |= 1UL<<0;

Delay(100ms);

ADC->CR2 |= 1UL<<0;

So the final setup be like: Enabling ADC 1

// Using ADC1 and pin PA1. which is at channel 1; RCC->APB2ENR ¦= RCC_APB2ENR_ADC1EN ¦ RCC_APB2ENR_AFIOEN; ADC1->CR2 = 0; // 1st of all channel and count must be set to proper value, then ADON need to set twitch. ADC1->SQR3 = 1; ADC1->CR2 ¦= ADC_CR2_ADON ¦ ADC_CR2_CONT; delay(500); ADC1->CR2 ¦= ADC_CR2_ADON;//enable adc and start continuous conversion

# GPIO pin config

The input pin need to set in analog input mode. Which is  CNF =00, MODE = 00;

# ADC_Check:

Checking ADC has completed conversion or not.

## ADC status register.

We need the bit 1 (EOC) which is end of conversion. If it is set, then last conversion is done and data is ready to scan.

if (ADC1->SR & 1UL<<1){

Read ( converted data);}

# Reading converted data (0 to 2^12)

## ADC regular Data register ADC->DR:

Data = ADC1->DR;

Converting the range (0 to 2^12) to (0 to 1000) to get better visual.

Value 2^12 (0xfff) is equal to 1000

Value 1 is equal to 1000/(2^12)

Value x is equal  to (x*1000)/(2^12);

Result = (data*1000)/0xfff;

Code:

//pa1 as analog input #include<stdint.h> #include<stm32f10x.h> void En_clock(void); void gpio_setup(void); void delay_ms(void); void delay(uint32_t count); void systick_config(void); uint8_t debounce(uint8_t last); void ADC_config(void); int main(void){ uint8_t lastb = 0, currentb=0; En_clock(); gpio_setup(); systick_config(); ADC_config(); uint16_t analog_data = 0; while(1){ currentb = debounce(lastb); //If conversion is done, read the data if(ADC1->SR & ADC_SR_EOC){ analog_data = ADC1->DR; analog_data = analog_data*1000/0xfff; delay(2000); } //checking loop main function is running GPIOA->ODR ¦= GPIO_ODR_ODR3; delay(20); GPIOA->ODR &= ~GPIO_ODR_ODR3; delay(20); } return 0; } void En_clock(void){ RCC->APB2ENR ¦= RCC_APB2ENR_AFIOEN ¦ RCC_APB2ENR_IOPAEN; } void gpio_setup(void){ //PA) as push button GPIOA->CRL &= ~(GPIO_CRL_CNF0 ¦ GPIO_CRL_MODE0); GPIOA->CRL ¦= GPIO_CRL_MODE0; // PA1 as analog in put cnf=00, mode =00; GPIOA->CRL &= ~(GPIO_CRL_CNF1 ¦ GPIO_CRL_MODE1); GPIOA->CRL ¦= 0UL; //PA5 as led output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF5 ¦ GPIO_CRL_MODE5); GPIOA->CRL ¦= GPIO_CRL_MODE5; //PA2 as push-pull output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF2 ¦ GPIO_CRL_MODE2); GPIOA->CRL ¦= GPIO_CRL_MODE2; //PA3 as push-pull output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF3 ¦ GPIO_CRL_MODE3); GPIOA->CRL ¦= GPIO_CRL_MODE3; //PA6 as push-pull output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF6 ¦ GPIO_CRL_MODE6); GPIOA->CRL ¦= GPIO_CRL_MODE6; } void systick_config(void){ SysTick->LOAD = 72000-1; SysTick->VAL = 0; SysTick->CTRL = SysTick_CTRL_CLKSOURCE ¦ SysTick_CTRL_ENABLE; } void delay_ms(void){ while(! ( SysTick->CTRL & SysTick_CTRL_COUNTFLAG)); } void delay(uint32_t count){ while(count--){ delay_ms(); }} uint8_t debounce(uint8_t last){ uint8_t current = (GPIOA->IDR & GPIO_IDR_IDR0)? 1 : 0; if ( last!= current){ delay(5); current = (GPIOA->IDR & GPIO_IDR_IDR0)? 1 : 0; } return current; } void ADC_config(void){ // Using ADC1 and pin PA1. which is at channel 1; RCC->APB2ENR ¦= RCC_APB2ENR_ADC1EN ¦ RCC_APB2ENR_AFIOEN; ADC1->CR2 = 0; ADC1->SQR3 = 1; ADC1->CR2 ¦= ADC_CR2_ADON ¦ ADC_CR2_CONT;//ADC power on delay(500); ADC1->CR2 ¦= ADC_CR2_ADON;//enable adc and start continuous conversion //delay(10); //ADC1->CR2 ¦= ADC_CR2_SWSTART; }

Turing on light if its dark,

A1 is analog input with photo registor…

A2, 3, 5 are output depending on registor value.

#include<stdint.h> #include<stm32f10x.h> void En_clock(void); void gpio_setup(void); void delay_ms(void); void delay(uint32_t count); void systick_config(void); uint8_t debounce(uint8_t last); void ADC_config(void); int main(void){ uint8_t lastb = 0, currentb=0; En_clock(); gpio_setup(); systick_config(); ADC_config(); uint16_t analog_data = 0; uint16_t max_val = 0x0C00; uint16_t min_val = 0x0700; while(1){ //If conversion is done, read the data if(ADC1->SR & ADC_SR_EOC){ analog_data = ADC1->DR; //analog_data = analog_data; } // Turn on 2 led if its dark, 1 if natural, off if bright if( analog_data>max_val){ //turn on A2 and A3, A5 GPIOA->ODR ¦= GPIO_ODR_ODR2; GPIOA->ODR ¦= GPIO_ODR_ODR3; GPIOA->ODR ¦= GPIO_ODR_ODR5; } else if ( analog_data<min_val){ //Turn on A2. but A3, and A5 turn off GPIOA->ODR ¦= GPIO_ODR_ODR2; GPIOA->ODR &= ~GPIO_ODR_ODR3; GPIOA->ODR &= ~GPIO_ODR_ODR5; } else { GPIOA->ODR ¦= GPIO_ODR_ODR2; GPIOA->ODR ¦= GPIO_ODR_ODR3; GPIOA->ODR &= ~GPIO_ODR_ODR5; } //checking loop main function is running GPIOA->ODR ¦= GPIO_ODR_ODR6; delay(20); GPIOA->ODR &= ~GPIO_ODR_ODR6; delay(20); } return 0; } void En_clock(void){ RCC->APB2ENR ¦= RCC_APB2ENR_AFIOEN ¦ RCC_APB2ENR_IOPAEN; } void gpio_setup(void){ //PA) as push button GPIOA->CRL &= ~(GPIO_CRL_CNF0 ¦ GPIO_CRL_MODE0); GPIOA->CRL ¦= GPIO_CRL_MODE0; // PA1 as analog in put cnf=00, mode =00; GPIOA->CRL &= ~(GPIO_CRL_CNF1 ¦ GPIO_CRL_MODE1); GPIOA->CRL ¦= 0UL; //PA5 as led output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF5 ¦ GPIO_CRL_MODE5); GPIOA->CRL ¦= GPIO_CRL_MODE5; //PA2 as push-pull output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF2 ¦ GPIO_CRL_MODE2); GPIOA->CRL ¦= GPIO_CRL_MODE2; //PA3 as push-pull output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF3 ¦ GPIO_CRL_MODE3); GPIOA->CRL ¦= GPIO_CRL_MODE3; //PA6 as push-pull output cnf =00, mode = 11; GPIOA->CRL &= ~(GPIO_CRL_CNF6 ¦ GPIO_CRL_MODE6); GPIOA->CRL ¦= GPIO_CRL_MODE6; } void systick_config(void){ SysTick->LOAD = 72000-1; SysTick->VAL = 0; SysTick->CTRL = SysTick_CTRL_CLKSOURCE ¦ SysTick_CTRL_ENABLE; } void delay_ms(void){ while(! ( SysTick->CTRL & SysTick_CTRL_COUNTFLAG)); } void delay(uint32_t count){ while(count--){ delay_ms(); }} uint8_t debounce(uint8_t last){ uint8_t current = (GPIOA->IDR & GPIO_IDR_IDR0)? 1 : 0; if ( last!= current){ delay(5); current = (GPIOA->IDR & GPIO_IDR_IDR0)? 1 : 0; } return current; } void ADC_config(void){ // Using ADC1 and pin PA1. which is at channel 1; RCC->APB2ENR ¦= RCC_APB2ENR_ADC1EN ¦ RCC_APB2ENR_AFIOEN; ADC1->CR2 = 0; ADC1->SQR3 = 1; ADC1->CR2 ¦= ADC_CR2_ADON ¦ ADC_CR2_CONT;//ADC power on delay(500); ADC1->CR2 ¦= ADC_CR2_ADON;//enable adc and start continuous conversion }